AICTE IDEA Lab Memento Board

v1.0

Project - Memento

From

www.aid-iot.com

Inordinate Industrial Innovations Pvt Ltd.

BP-5, MLU Plots, DDA, Bindapur, Dwarka, New Delhi, 110059

Table of Contents

1. Introduction	3

1.1 Board summary	3

1.2 What you need	4

2. Repository Layout	4

3. Hardware Design in KiCad	4

3.1 Opening the project	4

3.2 Creating a schematic symbol	4

3.3 Creating a footprint	5

3.4 Assigning footprints to symbols	5

Using the symbol properties	5

Using the assignment tool	6

3.5 Importing an image onto the silkscreen	6

Using the KiCad Image Converter	6

If the new footprint does not appear in the chooser	6

Putting a logo on the schematic	7

Cleaner outlines with logo2fp.py	7

Placing an image for reference only	7

3.6 Updating the PCB from the schematic	7

3.7 Component placement	8

3.8 Routing	8

3.9 Board outline and mounting holes	8

3.10 Electrical and design rule checks	9

Electrical Rules Check - the schematic	9

Design Rules Check - the board	9

3.11 Fabrication outputs	9

4. Project Details	9

4.1 Pin map	9

4.2 Bill of materials	10

4.3 Assembly notes	10

5. Programming the Board	10

5.1 Firmware layout	11

5.2 Setting the WiFi name and password	11

5.3 Method A - Arduino IDE	11

One-time setup	11

Every time	11

5.4 Method B - PlatformIO	11

6. Checking the Web Page	12

6.1 Finding the address	12

6.2 What the page shows	12

6.3 HTTP API	13

7. Troubleshooting	13

# 1. Introduction

The Memento is a keepsake circuit board built at the AICTE IDEA Lab, Poornima College of Engineering. It is a real working board as well as a souvenir: a Seeed Studio XIAO ESP32C3 module drives four addressable WS2812B RGB LEDs and reads air pressure and temperature from a BMP280 sensor module. The board joins a WiFi network and publishes the sensor readings on a small web page, which also lets you change the LED colour and animation from a phone or laptop.

The front of the board carries the AICTE IDEA Lab and Poornima College of Engineering logos in silkscreen.

Front copper, silkscreen and board outline

## 1.1 Board summary

| Item | Detail |
| --- | --- |
| Board size | 150 x 100 mm, 1.6 mm thick, 2 layers |
| Controller | Seeed Studio XIAO ESP32C3 (RISC-V, WiFi 4, BLE 5) |
| LEDs | 4 x WS2812B, 5050 PLCC-4, daisy chained |
| Sensor | GY-BMP280-3.3 module, I2C, pressure and temperature |
| Mounting | 4 x 4.25 mm holes, 13 mm in from every edge |
| Design tool | KiCad 10 |
| Firmware | Arduino / ESP32 core, builds in Arduino IDE or PlatformIO |

## 1.2 What you need

- KiCad 10 or newer, to open and edit the hardware design.

- Arduino IDE 2.x, or Visual Studio Code with the PlatformIO extension, to build the firmware.

- A USB-C cable that carries data, not a charge-only cable.

- A 2.4 GHz WiFi network. The ESP32-C3 does not support 5 GHz networks.

- A soldering iron and the parts in the bill of materials in section 4.2.

# 2. Repository Layout

The project is split into hardware, software and documentation:

memento/

hw/            KiCad project

memento.kicad_sch     schematic

memento.kicad_pcb     board layout

memento.kicad_pro     project file - open this one

memento.kicad_sym     project symbol library

memento.pretty/       project footprint library

assets/               logo source images

tools/                helper scripts

sw/            firmware

platformio.ini

memento_fw/

memento_fw.ino      the sketch

config.h            WiFi name and password

index_html.h        the web page

docs/          this manual

README.md

Note  The KiCad project carries its own symbol and footprint libraries, registered through the project-local sym-lib-table and fp-lib-table files. It opens on any machine with no extra library setup.

# 3. Hardware Design in KiCad

This section walks through how the board was built, in the order the work was done. Follow the same sequence to reproduce it or to make your own variant.

## 3.1 Opening the project

- Start KiCad.

- Choose File > Open Project.

- Browse to the hw folder and open memento.kicad_pro.

- The project manager lists the schematic and the PCB. Double-click either to open its editor.

## 3.2 Creating a schematic symbol

A symbol is the drawing of a part on the schematic, with its pins and pin numbers. KiCad ships thousands of symbols, but modules such as the XIAO ESP32C3 and the BMP280 breakout are not included, so they were drawn for this project and saved in the project library.

To create one:

- In the schematic editor, choose Tools > Symbol Editor.

- In the Libraries pane, select the project library (memento). If it is not listed, add it first with Preferences > Manage Symbol Libraries > Project Specific Libraries.

- Choose File > New Symbol. Give it a name and a reference designator prefix - U for modules and integrated circuits, D for LEDs, R for resistors.

- Draw the body with Place > Rectangle.

- Add pins with Place > Pin. For every pin set the number, the name and the electrical type. The number must match the footprint pad number exactly, because that is the only link between the two.

- Save the symbol.

Note  Electrical type matters. Power inputs marked as Power Input, outputs as Output, and so on, are what lets the Electrical Rules Check catch mistakes such as two outputs driven onto the same net.

The symbols in this project and their pin numbering:

| Symbol | Pins | Pin numbering |
| --- | --- | --- |
| XIAO_ESP32C3 | 14 | 1-7 down the left edge (D0-D6), 8-14 up the right edge (D7-D10, 3V3, GND, 5V) |
| WS2812B | 4 | 1 VDD, 2 DOUT, 3 VSS, 4 DIN |
| BMP280_Module | 6 | 1 VCC, 2 GND, 3 SCL, 4 SDA, 5 CSB, 6 SDO |

## 3.3 Creating a footprint

A footprint is the physical pattern of copper pads, silkscreen and courtyard that the part sits on. It must match the real part, so always work from the manufacturer drawing rather than from memory.

- In the PCB editor, choose Tools > Footprint Editor.

- Select the project library (memento) and choose File > New Footprint.

- Place pads with Place > Pad. Set the pad number to match the symbol pin number, and set the shape, size and drill.

- Draw the part outline on the F.Silkscreen layer, and the true body size on F.Fab.

- Draw a courtyard on F.CrtYd, slightly larger than the part, so KiCad can warn when parts overlap.

- Mark pin 1 clearly - a dot, a square pad or a chamfered corner.

- Save the footprint.

The footprints used here:

| Footprint | Type | Key dimensions |
| --- | --- | --- |
| XIAO-ESP32C3-Hybrid | Through-hole + SMD | 14 holes, 2.54 mm pitch, 15.24 mm between rows, 0.889 mm drill, plus castellation pads |
| WS2812B_PLCC4_5.0x5.0mm | SMD | 4 pads 1.5 x 0.9 mm, body 5.0 x 5.0 mm |
| BMP280_Module_GY_6P_15x11.5mm | Through-hole | 6 holes, 2.54 mm pitch, 1.0 mm drill, outline 15 x 11.5 mm |
| R_0805_2012Metric | SMD | Standard KiCad 0805 land pattern |

Note  The XIAO footprint is a hybrid: every pin has a plated through-hole for header mounting and an outboard SMD pad for the castellated edge, so the module can be socketed or soldered flat.

## 3.4 Assigning footprints to symbols

Each symbol on the schematic must be told which footprint to use. There are two ways:

### Using the symbol properties

- In the schematic, double-click the symbol.

- Click the Footprint field, then the library browser button at the right.

- Pick the footprint and click OK.

### Using the assignment tool

- In the schematic editor, choose Tools > Assign Footprints.

- The left pane lists libraries, the middle lists symbols, the right lists footprints.

- Select a symbol, then double-click the footprint to assign it.

- Click Apply, Save Schematic & Continue.

Note  If you set a Footprint field inside the library symbol itself, every new copy you place already has the correct footprint and no assignment step is needed. That is how the symbols in this project are set up.

## 3.5 Importing an image onto the silkscreen

The two logos on this board were imported from ordinary image files. KiCad includes a converter for exactly this.

### Using the KiCad Image Converter

- From the KiCad project manager, choose Tools > Image Converter.

- Click Load Source Image and select the picture. PNG works well. A high-contrast image gives the best result.

- Look at the Greyscale and Black and White preview tabs. Adjust the Threshold slider until the shape is clean and the detail you want is still visible.

- Set the output size, either by entering the DPI or by setting the size in millimetres. Check the aspect ratio lock so the image is not stretched.

- Under Output Format choose Footprint (.kicad_mod).

- Under Board Layer for Outline choose Front Silkscreen.

- Click Export to File and save the result inside your project footprint library folder, memento.pretty. KiCad only loads footprints that sit inside a folder listed in fp-lib-table, so a file saved anywhere else - the project root included - will not be found.

- In the PCB editor, choose Place > Place Footprints (shortcut A), pick the new footprint from the memento library and click to place it. On KiCad 8 and earlier the same command is named Add Footprint.

Two things to fix after placing a logo footprint:

- Open its properties and tick "Not in schematic" (the board-only attribute), together with "Exclude from bill of materials" and "Exclude from position files". Without this, Update PCB from Schematic will offer to delete the logo because it has no matching symbol.

- Check the smallest details against your fabricator's minimum silkscreen line width, usually about 0.15 mm. Fine lines below that fill in when printed, so scale the logo up if detail is being lost.

Note  The converter is monochrome. A colour logo becomes a single-colour silhouette: everything that is not background becomes ink, and enclosed background areas become gaps. Check the preview to confirm the result still reads correctly.

### If the new footprint does not appear in the chooser

Two causes account for nearly every case.

- The file was saved outside the library folder. The Image Converter reuses the last folder you exported to, which is often the project root. A loose .kicad_mod file is never picked up on its own; move it into memento.pretty and it appears in the chooser under the memento library.

- The library cache is stale. KiCad keeps an fp-info-cache file beside the project and does not always notice footprints added from outside the editor. Open Preferences > Manage Footprint Libraries and click OK to force a rescan, or close and reopen the PCB editor.

Note  KiCad names the library entry after the file, not after the name written inside it. A file saved as aid_iot_logo.kicad_mod is listed as aid_iot_logo even though the converter wrote (footprint "LOGO") on the first line. Rename the two to match so the library stays readable.

### Putting a logo on the schematic

The same converter can emit a schematic symbol instead: choose Symbol (.kicad_sym) under Output Format and export. A symbol library is a single .kicad_sym file rather than a folder, and it has to be registered before the symbol chooser will list it. In the schematic editor open Preferences > Manage Symbol Libraries, add the file on the Project Specific Libraries tab and give it a nickname; this writes a line into sym-lib-table beside the project.

A logo symbol has no pins. Mark it Exclude from bill of materials and Exclude from board in its properties so it never reaches the netlist.

### Cleaner outlines with logo2fp.py

The Image Converter works from a black and white threshold, which rounds off fine detail. This project also carries hw/tools/logo2fp.py, which traces the ink boundary directly and writes one filled polygon per outline, folding enclosed gaps back into their parent shape. It holds edges better at small sizes. Run it from the hw folder:

python tools/logo2fp.py assets/logo_poornima.png \

memento.pretty/Logo_Poornima_80x48.kicad_mod \

Logo_Poornima_80x48 80.0 400 F.SilkS 5:3

The arguments are the source image, the output path, the footprint name and the width in millimetres, followed by the optional trace resolution, target layer and box aspect ratio. Both logos already on this board were produced this way. The script needs numpy and Pillow.

### Placing an image for reference only

If you only want a picture as a guide - a mechanical drawing or a layout sketch - use Place > Place Reference Images in the PCB or schematic editor (named Add Image on KiCad 8 and earlier). Reference images are shown in the editor but are not plotted to the fabrication outputs, so they never appear on the finished board.

## 3.6 Updating the PCB from the schematic

The schematic is the source of truth. The board is brought into line with it rather than edited independently.

- Draw the circuit in the schematic editor. Place symbols with Place > Add Symbol, wire them with Place > Wire, and add power symbols for the supply nets.

- Annotate the sheet with Tools > Annotate Schematic so every part gets a unique reference designator.

- Run the Electrical Rules Check with Inspect > Electrical Rules Checker and clear anything real.

- Save the schematic.

- In the PCB editor choose Tools > Update PCB from Schematic, or press F8.

- Review the list of changes and click Update PCB. New footprints appear at the cursor for you to place.

Note  KiCad links each symbol to its footprint by a hidden unique identifier, not by the reference designator. That is why renaming a part does not break the link, and why a footprint added directly to the board with no symbol behind it must be marked board-only.

## 3.7 Component placement

Placement decides how easy the routing will be. Work from the fixed constraints outwards.

- Place the parts whose position is dictated by the enclosure or by the user first. Here the XIAO sits at the right-hand edge so the USB-C connector is reachable.

- Place the parts that must sit near something else. The pull-up resistors R2 and R3 sit next to the BMP280 module, and the series resistor R1 sits next to the XIAO data pin.

- Place the decorative elements - here the two logos and the text labels - and keep them clear of the parts.

- Spread the four WS2812B LEDs around the logo so the light is even.

- Use Edit > Properties or the Position fields to set exact coordinates where symmetry matters.

Useful tools while placing:

- Press M to move a part, R to rotate it, F to flip it to the other side of the board.

- The ratsnest - the thin lines showing what still needs connecting - shows when a move will make routing shorter.

- View > Grid, and the grid selector in the toolbar, keep parts on tidy coordinates.

- Inspect > Design Rules Checker reports courtyard overlaps if two parts are placed too close.

## 3.8 Routing

Routing draws the copper tracks that carry the signals.

- Select the copper layer to work on in the Layers pane - F.Cu for the front, B.Cu for the back.

- Choose Route > Route Single Track, or press X, then click a pad to start.

- Click to place corners and double-click to finish on the destination pad. The ratsnest line for that connection disappears once it is routed.

- Press V while routing to drop a via and continue on the other layer.

- Set track widths under File > Board Setup > Design Rules > Net Classes. Power nets are usually given wider tracks than signals.

- When everything is routed, run the Design Rules Checker and confirm there are no unconnected items left.

Guidance that applies to this board:

- Give the +5 V and GND tracks more width than the signal tracks. They carry the LED current, which rises with brightness.

- Keep the WS2812B data chain going in one direction: XIAO to D2, D2 to D3, D3 to D4, D4 to D5. Each LED regenerates the signal for the next.

- Keep the I2C pair SDA and SCL reasonably short and run them together.

- Leave clearance around the mounting holes so a screw head or standoff cannot touch copper.

## 3.9 Board outline and mounting holes

The board shape is drawn on the Edge.Cuts layer. Anything on that layer is cut by the fabricator.

- Select the Edge.Cuts layer.

- Draw the outline with Place > Line, Place > Rectangle or Place > Arc. It must form one closed shape with no gaps.

- For mounting holes, either place a circle on Edge.Cuts, or place a mounting-hole footprint from the KiCad MountingHole library if you want a plated hole with a pad.

- Check the result with View > 3D Viewer, which shows the board exactly as it will be cut.

This board is 150 x 100 mm with rounded inside corners and four mounting holes of 4.25 mm diameter, each set 13 mm in from both nearest edges so the pattern is symmetric about the centre of the board.

## 3.10 Electrical and design rule checks

Two separate checks must both be clean before sending the board out.

### Electrical Rules Check - the schematic

- In the schematic editor choose Inspect > Electrical Rules Checker.

- Click Run ERC.

- Work through each message. Common ones are unconnected pins, and power input pins with nothing driving them - which is normal until you add power flags.

### Design Rules Check - the board

- In the PCB editor choose Inspect > Design Rules Checker.

- Tick "Report all errors for tracks" and click Run DRC.

- Fix clearance violations, unconnected items and silkscreen overlaps.

- Pay attention to "unconnected items". Any number other than zero means part of the circuit is not routed.

Note  Set the clearance and track width rules to match your fabricator before routing, under File > Board Setup > Design Rules > Constraints. Fixing a board designed to the wrong rules is far more work than setting them first.

## 3.11 Fabrication outputs

When both checks are clean, produce the files the factory needs.

- Choose File > Fabrication Outputs > Gerbers. Select the layers and export to a gerbers folder.

- Choose File > Fabrication Outputs > Drill Files and export into the same folder.

- Zip the folder and send it to the fabricator.

- For assembly, also export the position file from File > Fabrication Outputs > Component Placement, and the bill of materials from the schematic editor with Tools > Generate BOM.

# 4. Project Details

## 4.1 Pin map

These are the actual connections in the schematic. The firmware uses exactly these pins.

| Signal | XIAO pin | GPIO | Connects to |
| --- | --- | --- | --- |
| WS2812B data | D0 | 2 | R1, then D2.DIN, then D3, D4, D5 in a chain |
| I2C SDA | D4 | 6 | U2 SDA, pulled up to 3V3 by R3 |
| I2C SCL | D5 | 7 | U2 SCL, pulled up to 3V3 by R2 |
| +5 V | 5V | - | VDD of all four WS2812B LEDs |
| +3.3 V | 3V3 | - | BMP280 VCC and the two I2C pull-ups |
| GND | GND | - | Every part |

Note  The BMP280 module runs from 3.3 V, not 5 V. The GY-BMP280-3.3 has no on-board regulator and the BMP280 is not 5 V tolerant, so connecting it to 5 V damages it.

## 4.2 Bill of materials

| Ref | Qty | Part | Package | Notes |
| --- | --- | --- | --- | --- |
| U1 | 1 | Seeed Studio XIAO ESP32C3 | 14-pin module, 2.54 mm | Headers or castellated mounting |
| U2 | 1 | GY-BMP280-3.3 module | 6-pin, 2.54 mm | Pressure and temperature, I2C |
| D2, D3, D4, D5 | 4 | WS2812B | 5050 PLCC-4 | Addressable RGB LED |
| R1 | 1 | 330 ohm | 0805 | Series resistor on the LED data line |
| R2, R3 | 2 | 4.7 k ohm | 0805 | I2C pull-ups to 3V3 |
| - | 4 | M4 screw and standoff | - | For the 4.25 mm mounting holes |
| - | 1 | USB-C cable | - | Must carry data, not charge only |

Note  The resistor value fields in the schematic are still at their default and read R_US. The values above are the intended ones - set them in the schematic before generating fabrication files or before ordering parts.

## 4.3 Assembly notes

- Solder the four WS2812B LEDs first. They are surface mount and easiest to reach while the board is otherwise empty. Watch the orientation - the cut corner or the marking on the package matches pin 1 on the footprint.

- Solder the three 0805 resistors.

- Fit a 7-pin header strip on each side for the XIAO, or solder the module directly onto the castellated pads.

- Fit a 6-pin header for the BMP280 module.

- Check for solder bridges before applying power, especially across the fine-pitch LED pads.

- Power the board over USB-C and confirm the LEDs light before going further.

# 5. Programming the Board

The firmware lives in the sw folder. It is written so that the same files open in the Arduino IDE and build under PlatformIO, with nothing to convert between them.

## 5.1 Firmware layout

| File | Purpose |
| --- | --- |
| sw/platformio.ini | PlatformIO settings. Points at the sketch folder so both tools share one copy of the code. |
| sw/memento_fw/memento_fw.ino | The sketch - setup, main loop, sensor reading, LED animation and web server. |
| sw/memento_fw/config.h | WiFi name and password, and the start-up LED settings. This is the file you edit. |
| sw/memento_fw/index_html.h | The web page, stored as text in program memory. |

## 5.2 Setting the WiFi name and password

Open sw/memento_fw/config.h and change these two lines:

#define WIFI_SSID       "YOUR_WIFI_NAME"

#define WIFI_PASSWORD   "YOUR_WIFI_PASSWORD"

If the board cannot join that network within 20 seconds it starts its own hotspot instead, so the page is still reachable:

#define AP_SSID         "memento-setup"

#define AP_PASSWORD     "memento123"

Connect a phone or laptop to that hotspot and open http://192.168.4.1/ to reach the page.

Note  The ESP32-C3 only joins 2.4 GHz networks. If your router broadcasts one name for both 2.4 GHz and 5 GHz, and the board will not connect, try a separate 2.4 GHz network name or a phone hotspot set to 2.4 GHz.

## 5.3 Method A - Arduino IDE

### One-time setup

- Open File > Preferences.

- In "Additional boards manager URLs" paste: https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

- Click OK, then open Tools > Board > Boards Manager.

- Search for esp32 and install "esp32 by Espressif Systems".

- Open Tools > Manage Libraries and install these four: Adafruit NeoPixel, Adafruit BMP280 Library, Adafruit Unified Sensor, Adafruit BusIO.

### Every time

- Open sw/memento_fw/memento_fw.ino. The Arduino IDE shows config.h and index_html.h as extra tabs, because they sit in the same folder as the sketch.

- Edit the WiFi name and password on the config.h tab.

- Choose Tools > Board > ESP32 Arduino > XIAO_ESP32C3.

- Plug in the board and choose its port under Tools > Port.

- Click Upload.

Note  If no port appears, hold the BOOT button on the XIAO, tap RESET, then release BOOT. The board enters its bootloader and the port appears. Also check the USB cable carries data - charge-only cables are a common cause.

## 5.4 Method B - PlatformIO

From the sw folder. PlatformIO downloads the toolchain and the libraries by itself on the first build.

pio run              build only

pio run -t upload    build and flash the board

pio device monitor   open the serial log at 115200 baud

In Visual Studio Code, open the sw folder and use the PlatformIO toolbar buttons at the bottom of the window instead of typing the commands.

# 6. Checking the Web Page

## 6.1 Finding the address

- Keep the board plugged into USB after uploading.

- Open the serial monitor - in the Arduino IDE, Tools > Serial Monitor; in PlatformIO, pio device monitor.

- Set the speed to 115200 baud.

- Press the RESET button on the XIAO so you see the start-up messages from the beginning.

You should see something like this:

memento starting

BMP280 found at 0x76

Joining "YOUR_WIFI_NAME" ....

Connected. Open http://192.168.1.42/  (or http://memento.local/)

HTTP server up on port 80

- Type that address into a browser on any device connected to the same network.

Note  http://memento.local/ works on most phones and on Windows, macOS and Linux machines with mDNS. If it does not resolve on your network, use the numeric address instead - it always works.

## 6.2 What the page shows

The page has two parts.

The BMP280 panel shows three live readings, refreshed every two seconds:

- Temperature, in degrees Celsius.

- Pressure, in hectopascal.

- Altitude, in metres. This is calculated from the pressure, so it is only as accurate as the sea-level pressure set in config.h.

If the sensor is not detected the panel turns amber and the readings show as dashes. The firmware keeps retrying every five seconds, so a loose module starts working again as soon as it is reseated, without a reset.

The WS2812B panel controls the four LEDs:

| Control | What it does |
| --- | --- |
| Solid | All four LEDs show the chosen colour |
| Rainbow | Colours cycle smoothly around the chain |
| Breathe | The chosen colour fades up and down |
| Chase | A bright dot runs around the chain |
| Off | All LEDs off |
| Colour | Colour picker, used by Solid, Breathe and Chase |
| Brightness | 0 to 255 |
| Speed | Milliseconds per animation step - lower is faster |

The line at the bottom of the page shows whether the board is on your WiFi or running its own hotspot, its address, the signal strength, how long it has been running, and the sensor address.

## 6.3 HTTP API

The page is driven by two simple endpoints, which you can also call yourself for testing or to connect the board to something else.

| Route | Purpose |
| --- | --- |
| GET / | The control page |
| GET /api/status | JSON with the sensor readings, LED state, network details and uptime |
| GET /api/led | Sets the LEDs. Parameters: mode, r, g, b, brightness, speed |

For example, to set all four LEDs to solid orange at medium brightness:

curl "http://memento.local/api/led?mode=solid&r=255&g=80&b=0&brightness=120"

And to read the sensor from a script:

curl http://memento.local/api/status

# 7. Troubleshooting

| Symptom | What to check |
| --- | --- |
| No serial port appears | Use a data USB cable. Hold BOOT, tap RESET, release BOOT to force the bootloader. |
| Upload fails part way | Close the serial monitor first - it holds the port open. Then retry. |
| Board never joins WiFi | The network must be 2.4 GHz. Check the name and password for typing mistakes and stray spaces. Watch the serial log for the reason. |
| Page will not open | Use the numeric address from the serial log rather than memento.local. Confirm your phone or laptop is on the same network. |
| BMP280 not detected | The serial log reports it at start-up. Check the 6-pin header solder joints, that VCC goes to 3V3, and that CSB is not pulled low. The firmware probes both 0x76 and 0x77. |
| Sensor readings look wrong | Altitude depends on SEA_LEVEL_HPA in config.h. Set it to your local sea-level pressure for a meaningful altitude. |
| LEDs stay dark | Check the brightness is not zero and the mode is not Off. Confirm 5 V reaches the first LED and that R1 is fitted. |
| Only the first LED works | The chain is broken after it. Check the DOUT to DIN joint between that LED and the next. |
| LED colours are wrong or flicker | The data line runs at 3.3 V into 5 V parts, which is below the datasheet threshold. It usually works, but a level shifter is the proper fix. |